Experiment 06:

Implement VLAN and VLAN Trunking protocols.

A traditional Layer 2 switch works primarily using MAC addresses.

Suppose we have:

Suppose PC1 wants to communicate with PC2.

PC1 sends an Ethernet frame containing something like:

Source MAC      = MAC of PC1

Destination MAC = MAC of PC2

The switch looks at the destination MAC address and decides where to forward the frame.

The switch maintains something called a: MAC Address Table.

For Example:

| MAC Address | Port |
| --- | --- |
| PC1 MAC | Fa0/1 |
| PC2 MAC | Fa0/2 |
| PC3 MAC | Fa0/3 |

So if a frame comes for PC2, the switch knows: "PC2 is connected to Fa0/2."

Therefore, it forwards the frame through Fa0/2.  This is the basic operation of a Layer 2 switch.

What is a Layer 2 Switch?

A Layer 2 switch is a network device that primarily operates at the Data Link Layer (Layer 2) of the OSI model.

It uses MAC addresses to forward Ethernet frames.

For example:

PC1

|

| Ethernet

|

Switch

|

PC2

The switch doesn't normally make routing decisions based on IP addresses.

Its main job is: Receive an Ethernet frame → examine the MAC address → forward the frame through the appropriate port.

The Cisco 2960 switch that we commonly use in Packet Tracer is primarily a Layer 2 switch.

Then what is a Layer 3 Switch?

A Layer 3 switch can perform both: Layer 2 Switching + Layer 3 routing.

So it can understand:

- MAC addresses

- IP addresses

If a device in VLAN 10 wants to communicate with a device in VLAN 20, a normal Layer 2 switch cannot route between these two networks.

A Layer 3 switch can. It can perform Inter-VLAN Routing.

| Feature | Layer 2 Switch | Layer 3 Switch |
| --- | --- | --- |
| Main OSI Layer | Layer 2 | Layer 2 + Layer 3 |
| Uses MAC address | Yes | Yes |
| Uses IP address for routing | No | Yes |
| Switching | Yes | Yes |
| Routing | No | Yes |
| Inter-VLAN Routing | Not by itself | Yes |
| Example | Cisco 2960 | Cisco 3560 |

Now what is a VLAN?

VLAN stands for: Virtual Local Area Network

The important word here is Virtual. Normally, suppose we have one physical switch:

Without VLANs, these computers are part of the same Layer 2 broadcast domain.

Now imagine that the organization wants:

HR Department

Sales Department

to be logically separated.

We could physically use two switches:

Instead, we can use one physical switch and logically divide it.

Now we have created two logical networks inside the same physical switch.

That is the basic idea of VLAN.

What does VLAN actually do?

Suppose:

VLAN 10 → HR

VLAN 20 → Sales

Then:

PC1 ── VLAN 10

PC2 ── VLAN 10

PC3 ── VLAN 20

PC4 ── VLAN 20

PC1 and PC2 belong to the same VLAN.

PC3 and PC4 belong to another VLAN.

Therefore:

PC1 → PC2

PC3 → PC4

PC1 → PC3

assuming no Layer 3 routing has been configured. This is exactly the concept we were observing earlier.

Why does VLAN separate devices?

This is an extremely important concept. A switch normally forwards Ethernet frames within a broadcast domain.

When we create VLANs, the switch logically separates its ports.

For example:

The switch treats VLAN 10 and VLAN 20 as separate Layer 2 networks.

Therefore, a broadcast from VLAN 10 does not normally cross into VLAN 20.

Problems in normal LAN.

| Problem in a Normal LAN | How VLAN Solves It | Why It Is Needed |
| --- | --- | --- |
| 1. All devices are part of the same broadcast domain | VLAN divides one physical LAN into multiple logical networks. | Reduces unnecessary broadcast traffic. |
| 2. Difficult to separate departments | Different departments can be placed in different VLANs. | For example, Accounts → VLAN 10, HR → VLAN 20, IT → VLAN 30. |
| 3. Security is reduced | Devices belonging to different VLANs are logically separated. | Users in one VLAN cannot directly communicate with another VLAN without routing. |
| 4. Broadcast traffic reaches many unnecessary devices | Broadcasts are normally limited to the same VLAN. | Improves network performance. |
| 5. Physical location determines network grouping | VLAN allows logical grouping regardless of physical location. | Two PCs in different rooms can belong to the same VLAN. |
| 6. Moving a user may require changing physical connections | The switch port can simply be assigned to the required VLAN. | Network management becomes easier. |
| 7. A large LAN becomes difficult to manage | A large LAN can be divided into smaller VLANs. | Makes the network easier to organize and manage. |
| 8. Departments may need separate networks | Each department can have its own VLAN. | Provides logical separation between departments. |
| 9. Broadcast storms can affect a large LAN | VLANs create separate broadcast domains. | A broadcast problem in one VLAN is less likely to affect other VLANs. |
| 10. One physical switch may need to support multiple networks | VLANs allow multiple logical networks on the same physical switch. | Saves hardware and reduces infrastructure cost. |

What is the advantage of this?

Imagine a college network with:

Students

Faculty

Administration

Accounts

Instead of putting everybody into one huge network, we can create:

VLAN 10 → Students

VLAN 20 → Faculty

VLAN 30 → Administration

VLAN 40 → Accounts

This gives us logical separation.

Even though they may physically use the same switch infrastructure, they are logically separated.

What are Access ports in Switches?

An access port is a switch port that normally belongs to one VLAN.

For example:

PC1

|

|

Fa0/1

|

Switch

Suppose we configure: Fa0/1 → VLAN 10

PC1 is therefore connected to VLAN 10.

Another PC:

PC2

|

|

Fa0/2

|

Switch

Suppose: Fa0/2 → VLAN 20

Then PC2 belongs to VLAN 20.

An access port is generally used to connect:

- PCs

- Printers

- Servers

- IP phones in appropriate configurations

- End-user devices

What are Trunk Ports?

Imagine we have two switches.

Suppose both switches have:

VLAN 10 → HR

VLAN 20 → Sales

We need to connect the two switches.

The question is: How can one physical cable carry traffic belonging to both VLAN 10 and VLAN 20?

We could use two cables:

But this is inefficient. Instead, we use a: Trunk Port.

A trunk port is a switch port that can carry traffic belonging to multiple VLANs over a single physical link.

For example:

One physical cable can carry traffic for multiple VLANs. This is the fundamental purpose of VLAN trunking.

| Access Port | Trunk Port |
| --- | --- |
| Normally carries one VLAN | Carries multiple VLANs |
| Usually connects end devices | Usually connects network devices |
| PC → Switch | Switch → Switch |
| Traffic is normally untagged toward an end device | VLAN information is carried across the trunk |
| Example: Fa0/1 assigned to VLAN 10 | Example: Gi0/1 carrying VLAN 10,20,30 |

But how does the receiving switch know which VLAN a frame belongs to?

This is the key problem that VLAN trunking solves. Suppose Switch 1 sends a frame across the trunk.

The frame needs some way to communicate: "I belong to VLAN 10."

Otherwise, switch 2 would not know whether the frame belongs to VLAN 10, VLAN 20, VLAN 30, etc.

This is where VLAN tagging comes in.

What is VLAN Tagging?

With Ethernet trunking, VLAN information can be added to the Ethernet frame.

The most important standard used for this is: IEEE 802.1Q or simple called as 802.1Q.

A VLAN tag contains information that allows the receiving switch to identify the VLAN.

Conceptually:

For example:

Frame

|

+---- VLAN ID = 10

The receiving switch sees VLAN ID 10 and knows: "This frame belongs to VLAN 10."

What is VLAN Trunking?

VLAN trunking is the process of carrying traffic from multiple VLANs over a single physical network link between network devices.

VLAN Trunking ≠ VTP

There are three different concepts that sound similar:

- VLAN Trunking: Carries multiple VLANs over one link.

- 802.1Q: The standard used to tag VLAN traffic on an Ethernet trunk.

- VTP (VLAN Trunking Protocol): But VTP does not mean "the protocol that carries multiple VLANs over a trunk."

VTP is used to distribute/manage VLAN configuration information among Cisco switches in a VTP domain.

Types of Trunking Protocols:

| Protocol | Type | Status/Use |
| --- | --- | --- |
| IEEE 802.1Q | Open IEEE standard | Common/current Ethernet VLAN trunking method |
| ISL (Inter-Switch-Link) | Cisco proprietary | Legacy/obsolete in modern networks |

We will use IEEE 802.1Q trunking.

Steps:

Step 1: Take two switches 2960 Cisco and take 4 PC’s.

Step 2: Use Copper-Straight Through cables to connect them.

PC1 → SW1 Fa0/1

PC2 → SW1 Fa0/2

PC3 → SW2 Fa0/1

PC4 → SW2 Fa0/2

Then connect two switches:

SW1 Gi0/1 → SW2 Gi0/1

The topology should look like this:

The link between SW1 and SW2 is going to become the trunk link.

Step 3:

Assign IP Addresses:

PC1:

IP Address:      192.168.10.11

Subnet Mask:     255.255.255.0

PC3:

IP Address:      192.168.10.12

Subnet Mask:     255.255.255.0

PC1 and PC3 are going to belong to: VLAN 10

So their IP addresses are also from: 192.168.10.0

PC2:

IP Address:      192.168.20.11

Subnet Mask:     255.255.255.0

PC4:

IP Address:      192.168.20.12

Subnet Mask:     255.255.255.0

VLAN 20.

Step 4:  Configuring VLAN on SW1 (Switch 1).

SW1 -> GO TO CLI.

Switch>

Enter Privileged Mode:

Type: Enable

Switch#

Enter Configuration Mode:

Type: Configure Terminal or Conf t

Switch(config)#

Step 5: Create VLAN 10 on SW1

Type: VLAN 10

Switch(config-vlan)#

Give it a name: name STUDENTS

Now VLAN 10 has been created.

VLAN 10 -> STUDENTS.

Step 6: Create VLAN 20 on SW1

Type: Exit

Then: VLAN 20

And

Name FACULTY

Now SW1 has:

VLAN 10 -> STUDENTS

VLAN 20 -> FACULTY

Step 7: Assigning PORTS to VLAN.

Creating a VLAN does not automatically put PCs into that VLAN.

We still have to tell the switch: “Fa0/1 belongs to VLAN 10”

And “Fa0/1 belongs to VLAN 20”

Step 8: Configure Fa0/1 as an Access Port.

Type: Exit

Now enter the interface: interface fa0/1

You should see: Switch(config-if)#

Now tell the switch that this is an access port:

Type: Switchport mode access

Then assign it to VLAN 10: switchport access vlan 10

So the complete configuration will be:

interface fa0/1

switchport mode access

switchport access vlan 10

By this we tell SW1 that fa0/1 -> Access Port -> VLAN 10.

Therefore, PC1- > Fa0/1 -> VLAN 10.

Step 9: Configure Fa0/2 for VLAN 20:

Type: Exit

interface fa0/2

then

switchport mode access

then

switchport access vlan 10.

Step 10: Repeat the same process for SW2.

Step 11: Now we have to configure the TRUNK.

This is the most important part of the experiment.

We have: SW1 Gi0/1 ================= Gi0/1 SW2

Currently, this is just a normal switch-to-switch connection.

We need to tell both switches:

"This link is going to carry multiple VLANs."

Therefore, we configure it as a trunk.

Step 12:

On SW1:

enable

configure terminal

then

interface gigabitEthernet 0/1

then

switchport mode trunk

Now Gig0/1 is now the trunk port.

Step 13: Repeat the same process for SW2.

Step 14:

Suppose PC1 sends a frame to PC3.

PC1 belongs to: VLAN 10

The frame reaches:

PC1

↓

SW1 Fa0/1

↓

VLAN 10

SW1 needs to send that frame to SW2.

The frame travels through:

SW1

↓

Gi0/1

↓

TRUNK

↓

Gi0/1

↓

SW2

Because it is crossing a trunk, VLAN information is carried using the trunking mechanism, typically IEEE 802.1Q.

SW2 receives the frame and knows: "This frame belongs to VLAN 10."

It then forwards it to:

VLAN 10

↓

Fa0/1

↓

PC3

Therefore, PC1 → PC3 works.

Step 15: Testing the configurations.

Ping PC1 to PC3, it should work as they belong to same VLAN 10.

Ping PC2 to PC4, they should work as they belong to same VLAN 20.

But what happens, when we ping PC1 to PC4?  It should fail in this topology.

Because: VLAN 10 ≠ VLAN 20

and we haven't configured a router or Layer 3 switch.

Step 16: Verify VLANs

Show vlan brief

Step 17: Verify the Trunk

show interfaces trunk

Step 18: Verify MAC Address Learning

show mac address-table

Complete Configuration for SW1:

enable

configure terminal

vlan 10

name STUDENTS

exit

vlan 20

name FACULTY

exit

interface fa0/1

switchport mode access

switchport access vlan 10

exit

interface fa0/2

switchport mode access

switchport access vlan 20

exit

interface gigabitEthernet 0/1

switchport mode trunk

exit

Complete Configuration for SW2:

enable

configure terminal

vlan 10

name STUDENTS

exit

vlan 20

name FACULTY

exit

interface fa0/1

switchport mode access

switchport access vlan 10

exit

interface fa0/2

switchport mode access

switchport access vlan 20

exit

interface gigabitEthernet 0/1

switchport mode trunk

exit